# DS2 4x4 24-Feature TCN-GCN Results Report

Temporal GCN and TCN-GCN results for NoC attack detection and malicious-router localization

Dataset: data/processed/graph_dataset/paper1_temporal_graphs_ds2_4x4_portfeat.npz

Input tensor: [S, 16 routers, 8 epochs, 24 features]

# Executive Summary

The 24-feature per-port directional dataset enables much stronger malicious-router localization than aggregate features.

The original placement split was useful but not perfectly balanced across attacker counts.

A balanced seed7 placement split was created so validation and test both contain 1-, 2-, and 3-attacker cases.

On the balanced split, TCN-GCN clearly outperforms Conv1D-GCN for graph-level detection and node-level localization.

Current best balanced result: TCN-GCN with graph F1 = 0.9641, node F1 = 0.9248, exact localization = 0.7723, and topK exact = 0.8996.

# Final Balanced Comparison

| Model | Split | Graph F1 | Node F1 | Exact Loc. | TopK Exact | Top4 All |
| --- | --- | --- | --- | --- | --- | --- |
| Conv1D-GCN | balanced seed7 | 0.8665 | 0.8274 tuned | 0.4168 tuned | 0.8733 | 0.9939 |
| TCN-GCN | balanced seed7 | 0.9641 | 0.9248 fixed | 0.7723 fixed | 0.8996 | 0.9870 |

Key conclusion: under the harder balanced unseen-placement split, TCN-GCN generalizes much better than Conv1D-GCN.

# 1. Pre-Balanced Placement Split Results

The original placement split had validation containing 1- and 2-attacker cases, while test contained 3-attacker cases. It is useful, but not perfectly balanced across attacker counts.

| Model | Best Epoch | Graph F1 | Node F1 | Exact Loc. | TopK Exact |
| --- | --- | --- | --- | --- | --- |
| Conv1D-GCN | 58 | 0.9990 | 0.9571 | 0.7791 | 0.8927 |
| TCN-GCN | 14 | 0.9836 | 0.9779 | 0.9059 | 0.9411 |

On this original split, Conv1D-GCN is strongest for graph-level detection, while TCN-GCN substantially improves exact localization.

# 2. Balanced Placement Split Results

The balanced placement seed7 split includes one 1-attacker, one 2-attacker, and one 3-attacker run in both validation and test.

## 2.1 Balanced Conv1D-GCN

Best epoch: 46

Fixed threshold 0.50: graph F1 = 0.8665, node F1 = 0.8232, exact localization = 0.3976.

Validation-selected threshold 0.60: node F1 = 0.8274, exact localization = 0.4168.

Top-k: top1_any = 0.9163, topK_exact = 0.8733, top4_all = 0.9939.

## 2.2 Balanced TCN-GCN

Best epoch: 31

Fixed threshold 0.50: graph F1 = 0.9641, node F1 = 0.9248, exact localization = 0.7723.

Validation-selected threshold 0.55: node F1 = 0.9235, exact localization = 0.7670.

Top-k: top1_any = 0.9156, topK_exact = 0.8996, top4_all = 0.9870.

Per-attacker-count topK_exact: 1 attacker = 0.7990, 2 attackers = 0.9517, 3 attackers = 0.9481.

# 3. Metric Comparison Table

| model | split | threshold_type | graph_f1 | node_precision | node_recall | node_f1 | exact_localization | top1_any | topK_exact | top4_all |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conv1D-GCN | pre_balanced | fixed_0.50 | 0.9990 | 0.9284 | 0.9876 | 0.9571 | 0.7791 | 0.9886 | 0.8927 | 0.9953 |
| TCN-GCN | pre_balanced | fixed_0.50 | 0.9836 | 0.9711 | 0.9849 | 0.9779 | 0.9059 | 0.9932 | 0.9411 | 0.9941 |
| Conv1D-GCN | balanced_seed7 | tuned_0.60 | 0.8665 | 0.8118 | 0.8436 | 0.8274 | 0.4168 | 0.9163 | 0.8733 | 0.9939 |
| TCN-GCN | balanced_seed7 | fixed_0.50 | 0.9641 | 0.9315 | 0.9182 | 0.9248 | 0.7723 | 0.9156 | 0.8996 | 0.9870 |
| TCN-GCN | balanced_seed7 | tuned_0.55 | 0.9641 | 0.9331 | 0.9142 | 0.9235 | 0.7670 | 0.9156 | 0.8996 | 0.9870 |

# 4. Hardware / RTL Interpretation

Conv1D-GCN remains the cleaner first RTL accelerator target because its temporal encoder is simpler.

TCN-GCN is the accuracy/localization-optimized reference model and motivates a stronger temporal encoder.

The next VLSI-facing experiment should be feature ablation to determine whether all 24 features are required or whether a smaller monitor feature set can preserve localization performance.

# 5. Recommended Next Steps

Commit this report package and all scripts used for balanced splitting/evaluation.

Run feature ablation: aggregate 4 features, port counts only, port IFD only, and all 24 features.

Run multi-seed balanced splits if time allows.

Begin fixed-point/quantization study for the Conv1D-GCN RTL baseline.

Report generated for reports/tcn-gcn-ds-24feature. Metrics are based on the logged outputs shared during the DS2 4x4 24-feature balanced split experiments.